18X（1/3〞）CVI高清一体机机芯式样书

#### 产品型号

FTZ18SH72-CVI     FTZ18SH108-CVI

#### 产品外观图

#### 规格参数

| 摄像机 |  |  |
| --- | --- | --- |
| 规格型号 | FTZ18SH72-CVI | FTZ18SH108-CVI |
| 分辨率 | 130万像素 1280（H）X 720（V） | 200万像素 1920（H）X1080（V） |
| 通讯方式 | RS232 |  |
| 通讯协议 | SONY VISCA |  |
| 接口 | 9P FPC/6PX1.25mm连接器 |  |
| 传感器类型 | 1/3" CMOS |  |
| 扫描方式 | 逐行扫描 |  |
| 日夜转换模式 | 彩色/黑白/自动 |  |
| 最低照度 | 彩色：0.1Lux 黑白：0.01Lux |  |
| 同步方式 | 内同步 |  |
| 视频输出 | HD-CVI(1Vp-p 75欧) |  |
| 信噪比 | ≥50dB（AGC OFF） |  |
| 镜头 |  |  |
| IRCUT | IRCUT双滤自动切换 |  |
| 自动光圈 | 支持 |  |
| 光学镜头 | 18X， f=4.7mm～84.6mm F1.6(W)～2.7(T) |  |
| 视场角 | H：54.8°（W）～3.4°（T） V：41°（W）～2.5°（T） |  |
| 功能 |  |  |
| 曝光模式 | 自动/手动 |  |
| 白平衡 | 自动/室内/室外/手动 |  |
| 聚焦模式 | 自动/手动 |  |
| 增益控制 | 自动/手动 |  |
| 图片效果 | 自动/彩色/黑白/负片 |  |
| 电子放大 | —— |  |
| 背光补偿 | 开/关 |  |
| 图像冻结 | —— |  |
| 镜像功能 | 支持（水平镜像+垂直镜像） |  |
| 图像翻转 | 支持 |  |
| 黑屏功能 | —— |  |
| 宽动态 | 数字宽动态 |  |
| 数字降噪 | 2D数字降噪 |  |
| 电子快门 | 1/30s~1/10,000s |  |
| 数字慢快门 | —— |  |
| 对比度 | 可调 |  |
| 饱和度 | 可调 |  |
| 透雾功能 | 支持 |  |
| 移动侦测 | —— |  |
| 隐私遮蔽 | —— |  |
| 边缘照度补偿 | 支持 |  |
| 一般规范 |  |  |
| 外型尺寸 | 50（W）* 60（H）* 90（L） mm |  |
| 工作温度/湿度 | ﹣10℃～50℃，10％RH～60％RH |  |
| 保存温度/湿度 | ﹣20℃～60℃，10％RH～80％RH |  |
| 电源供应 | DC 12V±10% |  |
| 功耗 | 5W MAX |  |
| 净重 | 270g |  |

#### 引脚定义

9P FPC的接口定义是：(第一脚如上图所示位置)

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RXD | TXD | GND | DC12V | GND | CVI-OUT | GND | NC | GND |

6Px1.25mm连接器的接口定义是：(第一脚如上图所示位置)

| 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- |
| CVI- OUT | GND | RXD | TXD | GND | DC12V |

## 菜单

MENU: 菜单

第一页菜单：

LANG(语言)                 EN

ADDR(地址)                 01

BAUD(波特率)               9600

OSD-DISPOSD(显示)          ON/OFF

OEM-ZOOM(倍数最大值)       18

FOCUS(聚焦方式)            AF（自动）/MF（手动）/TRIG（触发）

D&NICR(切换)               AUTO（自动）/COL（彩色）/BW（黑白）

D&N-LV(彩转黑切换值)       0～31

N&D-LV(黑转彩切换值)       0～31

MIRROR(镜像)  	          OFF(关)/LR(左右)/UD(上下)/FLIP(翻转)

OUTPUT(视频输出格式)       1080P@25/1080P@30/720P@50/720P@60/720P@25 /720P@30

第二页菜单：

IRIS(光圈)               AUTO(自动)

WB(白平衡)               ATW(自动)/FULL/HOLD

WDR(宽动态)              ON/OFF

AGC(自动增益控制)        0-31

2D-NR(2D降噪)            0～5

BRIGHT(亮度)	           0-31

CONTRAST(对比度)	        0-31

SATURATION(饱和度)       0-31

SHARPNESS(锐度)          0-31

RESET(复位)

SAVE-EXIT(保存退出)

| CAM_Display Menu | On | 8x 01 04 DF 01 FF | 调用打开机芯菜单，必须在TITLE关闭情况下才有效。如果原来TITLE打开，那么发完TITLE关闭命令有需延时200ms以上才能发菜单打开命令 |
| --- | --- | --- | --- |
|  | Off | 8x 01 04 DF 00 FF | 保存退出 |
| MenuIng | 8x 09 04 DF FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |

操作：

菜单光标上下：  ZOOM TELE命令   ZOOM WIDE命令 （见标准协议命令）

菜单光标左右：  FOCUS NEAR命令  FOCUS FAR命令  （见标准协议命令）

菜单上下左右连续操作间隔需200ms以上

翻页操作： 在第一页最后一行光标向下则翻到第二页第一行

在第二页第一行光标向上则翻到第一页最后一行

ADDR(地址)，(BAUD)波特率 OUTPUT（视频输出格式）修改不立即生效，在退出菜单后才生效。

ADDR(地址) 和 (BAUD)波特率 默认锁定，不可修改

菜单中FOCUS 模式的TRIG为变倍触发（只在变倍完成后会有一次聚焦，然后不再对焦）

菜单复位功能： 进入菜单第二页，光标上下移动到“复位”处，NEAR/FAR进行复位

复位后 地址（ADDR）, 波特率(BAUD)和视频输出格式（OUTPUT）会保持进入菜单之前的值，其他的数据将回复到出厂默认值。

同时会退出菜单。

SONY协议高清命令列表

Command List

## （注明：命令中红色标记项为可选项，根据产品型号对应功能。）

#### Function Command List

饱和度设置（扩展命令）

|  | Reset | 8x 01 04 D0 00 FF | 饱和度设置 |
| --- | --- | --- | --- |
|  | Up | 8x 01 04 D0 02 FF |  |
| CAM_SATURATION | Down | 8x 01 04 D0 03 FF |  |
|  | Direct | 8x 01 04 D0 00 00 0p 0q FF | pq: 饱和度值(0x00~0x1f) 默认值：0x0D |

对比度设置（扩展命令）

|  | Reset | 8x 01 04 D1 00 FF | 对比度设置 |
| --- | --- | --- | --- |
|  | Up | 8x 01 04 D1 02 FF |  |
| CAM_CONTRAST | Down | 8x 01 04 D1 03 FF |  |
|  | Direct | 8x 01 04 D1 00 00 0p 0q FF | pq: 饱和度值(0x00~0x1f) 默认值：0x0C |

扩展查询命令

| CAM_ SaturationPosInq （扩展命令） | 8x 09 04 D0 FF | y0 50 00 00 0p 0q FF | pq: Saturation Position 饱和度值 |
| --- | --- | --- | --- |
| CAM_ContrastPosInq （扩展命令） | 8x 09 04 D1 FF | y0 50 00 00 0p 0q FF | pq: Contrast Position 对比度值 |

标准协议命令：

| SHANY V | Command Set | Command | Command Packet | Comments |
| --- | --- | --- | --- | --- |
| OK | AddressSet | Broadcast | 88 30 01 FF | Address setting |
| OK | IF_Clear | Broadcast | 88 01 00 01 FF | I/F Clesr |
| X | CommandCancel | - | 8x 2p FF | p: Socket No. (=1 or 2) |
| OK |  | On | 8x 01 04 00 02 FF | Power ON/OFF |
| OK | CAM_Power | Off | 8x 01 04 00 03 FF |  |
| OK |  | Stop | 8x 01 04 07 00 FF |  |
| OK |  | Tele(Standard) | 8x 01 04 07 02 FF |  |
| OK |  | Wide(Standard) | 8x 01 04 07 03 FF |  |
| OK | CAM_Zoom | Tele(Variable) | 8x 01 04 07 2p FF | 功能和上面一样 p=0～4 (Low) p = 5~7 (High) |
| OK |  | Wide(Variable) | 8x 01 04 07 3p FF |  |
| OK |  | Direct | 8x 01 04 47 0p 0q 0r 0s FF | pqrs: Zoom Position自动聚焦下可以单独发这个命令即有效，手动聚焦下必须和Focus Direct一起下发才有效，且zoom Direct 和 Focus Direct 命令之间间隔小于600ms， 如果需要发zoom focus两个坐标，建议用后面直接发两个坐标命令(8x 01 04 47 0p 0q 0r 0s 0t 0u 0v 0w FF) |
| X |  | On | 8x 01 04 06 02 FF | Digital zoom ON/OFF |
| X |  | Off | 8x 01 04 06 03 FF |  |
| X |  | Combine Mode | 8x 01 04 36 00 FF | Optical/Digital Zoom Combined |
| X |  | Separate Mode | 8x 01 04 36 01 FF | Optical/Digital Zoom Separate |
| X | CAM_DZoom | Stop | 8x 01 04 06 00 FF |  |
| X |  | Tele(Variable) | 8x 01 04 06 2p FF | p=0 (Low) to 7 (High) |
| X |  | Wide(Variable) | 8x 01 04 06 3p FF |  |
| X |  | x1/Max | 8x 01 04 06 10 FF | x1/MAX Magnification |
| X |  | Direct | 8x 01 04 46 00 00 0p 0q FF | pq: D-Zoom Position |
| SHANY V | Command Set | Command | Command Packet | Comments |
| OK |  | Stop | 8x 01 04 08 00 FF |  |
| OK |  | Far(Standard) | 8x 01 04 08 02 FF |  |
| OK |  | Near(Standard) | 8x 01 04 08 03 FF |  |
| X |  | Far(Variable) | 8x 01 04 08 2p FF | 功能和上面一样 |
| X |  | Near(Variable) | 8x 01 04 08 3p FF |  |
| X |  | Direct | 8x 01 04 48 0p 0q 0r 0s FF | pqrs: Focus Position 此命令必须和zoom direct一起下发才有效,且zoom Direct 和 Focus Direct 命令之间间隔小于600ms，如果需要发zoom focus两个坐标，建议用后面直接发两个坐标命令(8x 01 04 47 0p 0q 0r 0s 0t 0u 0v 0w FF) |
| OK | CAM_Focus | Auto Focus | 8x 01 04 38 02 FF | AF ON/OFF |
| OK |  | Manual Focus | 8x 01 04 38 03 FF |  |
| OK |  | Auto/Manual | 8x 01 04 38 10 FF |  |
| OK |  | One Push Trigger | 8x 01 04 18 01 FF | One Push AF Trigger |
| X |  | Infinity | 8x 01 04 18 02 FF | Forced infinity |
| X |  | Near Limit | 8x 01 04 28 0p 0q 0r 0s FF | pqrs: Focus Near Limit Position |
| X |  | Normal | 8x 01 04 58 02 FF | AF Sensitivity High/Low |
| X | AF Sensitivity | Low | 8x 01 04 58 03 FF |  |
| X |  | Normal AF | 8x 01 04 57 00 FF | AF Mode |
| X |  | Interval AF | 8x 01 04 57 01 FF |  |
| OK | CAM_AFMode | Zoom Trigger AF | 8x 01 04 57 02 FF |  |
| X |  | Active/Interval Time | 8x 01 04 27 0p 0q 0r 0s FF | pq: Movement Time, rs: Interval |
| X |  | Standard | 8x 01 04 11 00 FF | FOCUS IR compensation data |
| X | CAM_IRCorrection | IR Light | 8x 01 04 11 01 FF |  |
| OK | CAM_ZoomFocus | Direct | 8x 01 04 47 0p 0q 0r 0s 0t 0u 0v 0w FF | pqrs: Zoom Position tuvw: Focus Position |
| X |  | Lens | 8x 01 04 19 01 FF | Lens Initialization Start |
| X | CAM_Initialize | Comp Scan | 8x 01 04 19 02 FF | Correction of CCD pixel blemishes |
| OK |  | camera | 8x 01 04 19 03 FF | Camera reset |
| X |  | Auto | 8x 01 04 35 00 FF | Normal Auto |
| X |  | Indoor | 8x 01 04 35 01 FF | Indoor mode |
| X |  | Outdoor | 8x 01 04 35 02 FF | Outdoor mode |
| X |  | One Push WB | 8x 01 04 35 03 FF | One Push WB mode |
| X |  | ATW | 8x 01 04 35 04 FF | Auto Tracing White Balance |
| X |  | Manual | 8x 01 04 35 05 FF | Manual Control mode |
| X | CAM_WB | One Push Trigger | 8x 01 04 10 05 FF | One Push WB Trigger |
| X |  | Outdoor Auto | 8x 01 04 35 06 FF | Outdoor auto |
| X |  | Sodium Lamp Auto | 8x 01 04 35 07 FF | Auto including sodium lamp source |
| X |  | Sodium Lamp | 8x 01 04 35 08 FF | Sodium lamp source fixed mode |
| X |  | CRS | 8x 01 04 35 09 FF | Color rolling suppression mode |
| X |  | Reset | 8x 01 04 03 00 FF | Manual Control of R Gain |
| X |  | Up | 8x 01 04 03 02 FF |  |
| X | CAM_RGain | Down | 8x 01 04 03 03 FF |  |
| X |  | Direct | 8x 01 04 43 00 00 0p 0q FF | pq: R Gain |
| X |  | Reset | 8x 01 04 04 00 FF | Manual Control of B Gain |
| X |  | Up | 8x 01 04 04 02 FF |  |
| X | CAM_BGain | Down | 8x 01 04 04 03 FF |  |
| X |  | Direct | 8x 01 04 44 00 00 0p 0q FF | pq: B Gain |
| X |  | Full Auto | 8x 01 04 39 00 FF | Automatic Exposure mode |
| X |  | Manual | 8x 01 04 39 03 FF | Manual Control mode |
| SHANY V | Command Set | Command | Command Packet | Comments |
| OK | CAM_AE | AE MODE | 8x 01 04 39 XX FF | 00: AE AUTO 03: AE MANU |
| X |  | Iris Priority | 8x 01 04 39 0B FF | Iris Priority Automatic Exposure mode |
| X |  | Bright | 8x 01 04 39 0D FF | Bright Mode (Manual control) |
| X |  | Auto | 8x 01 04 5A 02 FF | Auto Slow Shutter ON/OFF |
| X | CAM_SlowShutter | Manual | 8x 01 04 5A 03 FF |  |
| OK |  | Reset | 8x 01 04 0A 00 FF | Shutter Setting |
| OK |  | Up | 8x 01 04 0A 02 FF |  |
| OK | CAM_Shutter | Down | 8x 01 04 0A 03 FF |  |
| X |  | Direct | 8x 01 04 4A 00 00 0p 0q FF | pq: Shutter Position |
| X |  | Reset | 8x 01 04 0B 00 FF | Iris Setting |
| X |  | Up | 8x 01 04 0B 02 FF |  |
| X | CAM_Iris | Down | 8x 01 04 0B 03 FF |  |
| X |  | Direct | 8x 01 04 4B 00 00 0p 0q FF | pq: Iris Position |
| OK |  | Reset | 8x 01 04 0C 00 FF | Gain Setting |
| OK |  | Up | 8x 01 04 0C 02 FF |  |
| OK | CAM_Gain | Down | 8x 01 04 0C 03 FF |  |
| OK |  | Direct | 8x 01 04 4C 00 00 0p 0q FF | pq: Gain Position |
| X |  | Gain Limit | 8x 01 04 2C 0p FF | p: Gain Position |
| OK |  | Reset | 8x 01 04 0D 00 FF | Bright Setting |
| OK |  | Up | 8x 01 04 0D 02 FF |  |
| OK | CAM_Bright | Down | 8x 01 04 0D 03 FF |  |
| OK |  | Direct | 8x 01 04 4D 00 00 0p 0q FF | pq: Bright Position(00~0x1f) ; default: 0x00 |
| X |  | On | 8x 01 04 3E 02 FF | Exposure Compensation ON/OFF |
| X |  | Off | 8x 01 04 3E 03 FF |  |
| OK |  | Reset | 8x 01 04 0E 00 FF | Exposure Compensation Amount Setting |
| OK | CAM_ExpComp | Up | 8x 01 04 0E 02 FF |  |
| OK |  | Down | 8x 01 04 0E 03 FF |  |
| OK |  | Direct | 8x 01 04 4E 00 00 0p 0q FF | pq: ExpComp Position(00~0x0f) 功能与亮度一样 |
| OK |  | On | 8x 01 04 33 02 FF | Back Light Compensation ON/OFF |
| OK | CAM_Backlight | Off | 8x 01 04 33 03 FF |  |
| X |  | On | 8x 01 04 59 02 FF | Spot Automatic Exposure Setting |
| X | CAM_SpotAE | Off | 8x 01 04 59 03 FF |  |
| X |  | Position | 8x 01 04 29 0p 0q 0r 0s FF | pq: X (0 to F), rs: Y (0 to F) |
| X | CAM_AE_Response | DIRECT | 8x 01 04 5D pp FF | Automatic Exposure Response Setting (01 to 30) |
| OK | CAM_WD | On | 8x 01 04 3D 02 FF | Wide-D ON/OFF |
| OK |  | Off | 8x 01 04 3D 03 FF |  |
| X |  | AutoOnOff | 8x 01 04 3D 00 FF | Wide dynamic ON/OF auto |
| X |  | On(RatioFix) | 8x 01 04 3D 01 FF | Fixed exposure ratio mode |
| X |  | On(Dver Compati) | 8x 01 04 3D 04 FF | Dver operation |
| X |  | Refresh | 8x 01 04 10 0D FF | Wide dynamic restart |
| SHANY V | Command Set | Command | Command Packet | Comments |
| X |  | Set Parameter | 8x 01 04 2D 0p 0q 0r 0s 0t 0u 00 00 FF | p: Screen display 0: Combined image, 1: Long/short division, 2: Long-time, 3: Short-time q: Detection sensitivity (0: L 1: M 2: H) r: Blocked-up shadow correction level (0: L 1: M 2: H 3: S) s: Blown-out highlight correction level (0: L 1: M 2: H) tu: Exposure ratio of short exposure (x1 to x150) |
| X |  | On | 8x 01 04 3B 02 FF | Wide dynamic auto switching alarm ON/OFF |
| X |  | Off | 8x 01 04 3B 03 FF |  |
| X | CAM_WDAlarmReply |  | y0 07 04 3B 02 FF | Wide dynamic OFF → ON |
| X |  | (Reply) | y0 07 04 3B 03 FF | Wide dynamic ON → OFF |
| OK |  | Reset | 8x 01 04 02 00 FF | Aperture Control |
| OK |  | Up | 8x 01 04 02 02 FF |  |
| OK | CAM_Aperture | Down | 8x 01 04 02 03 FF |  |
| OK |  | Direct | 8x 01 04 42 00 00 0p 0q FF | pq: Aperture Gain (0～0x0F) |
| X |  | On | 8x 01 04 52 02 FF | High-Resolusion Mode ON/OFF |
| X | CAM_HR | Off | 8x 01 04 52 03 FF |  |
| OK | CAM_NR | – | 8x 01 04 53 0p FF | p: NR Setting (0: OFF, level 1 to 5) |
| OK |  | On | 8x 01 04 61 02 FF | Mirror Image ON/OFF |
| OK | CAM_LR_Reverse | Off | 8x 01 04 61 03 FF |  |
| X |  | On | 8x 01 04 62 02 FF | Still Image ON/OFF |
| X | CAM_Freeze | Off | 8x 01 04 62 03 FF |  |
| OK |  | Off | 8x 01 04 63 00 FF | Picture Effect Setting |
| OK | CAM_PictureEffect | Neg.Art | 8x 01 04 63 02 FF |  |
| OK |  | B&W | 8x 01 04 63 04 FF |  |
| OK |  | On | 8x 01 04 66 02 FF | Picture flip ON/OFF |
| OK | CAM_PictureFlip | Off | 8x 01 04 66 03 FF |  |
| OK |  | On | 8x 01 04 01 02 FF | Infrared Mode ON/OFF |
| OK | CAM_ICR | Off | 8x 01 04 01 03 FF |  |
| OK |  | On | 8x 01 04 51 02 FF | Auto dark-field mode On/Off |
| OK | CAM_AutoICR | Off | 8x 01 04 51 03 FF |  |
| X |  | Threshold | 8x 01 04 21 00 00 0p 0q FF | pq: ICR ON → OFF threshold level |
| X |  | On | 8x 01 04 31 02 FF | Auto ICR switching Alarm ON/OFF |
| X | CAM | Off | 8x 01 04 31 03 FF |  |
| X | _AutoICRAlarmReply | (Reply) | y0 07 04 31 02 FF | ICR OFF → ON |
| X |  |  | y0 07 04 31 03 FF | ICR ON → OFF |
| OK |  | Reset | 8x 01 04 3F 00 0p FF | p: Memory Number (=0 to 5) |
| OK | CAM_Memory | Set | 8x 01 04 3F 01 0p FF |  |
| OK |  | Recall | 8x 01 04 3F 02 0p FF |  |
| OK |  | On | 8x 01 04 15 02 FF (8x 01 06 06 02 FF) | Display ON/OFF 机芯自带倍数显示开关。必须在菜单和TITLE都关闭的情况下才有倍数显示 |
| OK | CAM_Display | Off | 8x 01 04 15 03 FF (8x 01 06 06 03 FF) |  |
| SHANY V | Command Set | Command | Command Packet |  |
| OK |  | On/Off | 8x 01 04 15 10 FF (8x 01 06 06 10 FF) |  |
| OK |  | Title Set1 | 8x 01 04 73 00 mm nn pp qq 00 00 00 00 00 00 FF | L: Line Number, nn: H-Position pp: Color, qq: Blink |
| OK |  | Title Set2 | 8x 01 04 73 01 mm nn pp qq rr ss tt uu vv ww FF | mnpqrstuvw: Display character setting (1st to 10th character) |
| OK | CAM_Title | Title Set3 | 8x 01 04 73 02 mm nn pp qq rr ss tt uu vv ww FF | mnpqrstuvw: Display character setting (11th to 20th character) |
| OK |  | Title Clear | 8x 01 04 74 00 FF | Clear title setting |
| OK |  | On | 8x 01 04 74 02 FF | Title display ON/OFF |
| OK |  | Off | 8x 01 04 74 03 FF |  |
| OK |  | Title Set1 | 8x 01 04 73 1L 00 nn pp qq rr 00 00 00 00 00 FF | L: Line Number, nn: H-position pp: Color, qq: Blink rr: Opening Title颜色和闪烁无功能，本命令仅仅需要设置一次，所有行都是使用一个坐标，（如果不用设置坐标，则本命令可以不发送） 本命令无效，可不发此命令 |
| OK |  | Title Set2 | 8x 01 04 73 2L mm nn pp qq rr ss tt uu vv ww FF | L: Line Number, mnpqrstuvw: Setting of characters (1 to 10) |
| OK | CAM_MultiLineTitle | Title Set3 | 8x 01 04 73 3L mm nn pp qq rr ss tt uu vv ww FF | L: Line Number, mnpqrstuvw: Setting of characters (11 to 20) 如果有多行title，如果此时TITLE是打开的，那么每行title set 之间需要延时180ms以上，如果此时TITLE是关闭的，那么那么每行title set 之间需要延时40ms即可 |
| OK |  | Title Clear | 8x 01 04 74 1p FF | Title Setting clear (p: 0 to a, f= all lines) 发完此命令需延时240ms以上，确保显示不出现混乱 |
| OK |  | On | 8x 01 04 74 2p FF | Title display On/Off (0 to a, f= all lines) title on/title off两个命令都需要在发完后需延时240ms以上，确保显示不出现混乱 |
| OK |  | Off | 8x 01 04 74 3p FF |  |
|  | 关于title做菜单说明 |  |  | 如果用title做菜单，有两种方式，建议采样方式一 方式一：(推荐使用此方式) 菜单等待所有数据发送完成后一次性显示出来 1. 发送Title Off（8x 01 04 74 3f FF），不需要发送Title clear,以减少 刷新时间(因为发送Title clear又需要延时240ms) 2. Title Off后，延时240ms以上 3. 发送Title Set1，延时10ms （如果不用设置坐标，则本命令可以不发送）TITLE Set1 可以不发 4. 循环多行发送Title Set2和Title Set3， 每行完（即Title Set3后）延时40ms左右 5. 发送Title On（8x 01 04 74 2f FF）打开显示，完成菜单显示。 此时至少需要延时240ms以上才能对Title操作 6. 如果在打开Title on情况下需要单独修改其中一行Title内容，则发送Title Set2和Title Set3，然后至少延时180ms后才能有其他title操作 如果方式二： 采用逐行刷新方式，那么刷新时间会比方式一长，实现方式如下 1. 发送Title On（8x 01 04 74 2f FF），如果title已经打开那么命令不 需要发送 2. 发送了title on，则需要延时240ms以上 3. 发送Title clear（8x 01 04 74 1f FF）， 4. Title clear后，延时240ms以上 5. 发送Title Set1，延时10ms（如果不用设置坐标，则本命令可以不发送） TITLE Set1 可以不发 6. 循环多行发送Title Set2和Title Set3， 每行完（即Title Set3后）延时180ms以上，完成菜单显示 7. 如果在打开Title on情况下需要单独修改其中一行Title内容，则发送Title Set2和Title Set3，然后至少延时180ms后才能有其他title操作 |
| X |  | On | 8x 01 04 75 02 FF | Muting ON/OFF |
| X | CAM_Mute | Off | 8x 01 04 75 03 FF |  |
| X |  | On/Off | 8x 01 04 75 10 FF |  |
| OK | CAM_RegisterValue |  | 8x 01 04 24 mm 0p 0q FF | mm: Register NO pp: Register Value 见后面列表 |

| SHANY V | Command Set | Command | Command Packet | Comments |
| --- | --- | --- | --- | --- |
| X |  | SetMask | 8x 01 04 76 mm nn 0r 0r 0s 0s FF | mm: Mask Settings nn 00: Modify, 01: New rr: W, ss:H |
| X |  | Display | 8x 01 04 77 pp pp pp pp FF | Mask Display ON/OFF pp pp pp pp: Mask Settings (0:OFF, 1:ON) |
| X | CAM_PrivacyZon | SetMaskColor | 8x 01 04 78 pp pp pp pp qq rr FF | pp pp pp pp: Mask Color Settings qq: Color Setting when 0 is selected rr: Color Setting when 1 is selected |
| X | e | SetPanTiltAngle | 8x 01 04 79 0p 0p 0p 0q 0q 0q FF | Pan/Tilt Angle Settings ppp: Pan qqq: Tilt |
| X |  | SetPTZMask | 8x 01 04 7B mm 0p 0p 0p 0q 0q 0q 0r 0r 0r 0r FF | Pan/Tilt/Zoom Settings for Mask ppp: Pan, qqq: Tilt, rrrr: Zoom |
| X |  | Non_InterlockM ask | 8x 01 04 6F mm 0p 0p 0q 0q 0r 0r 0s 0s FF | mm: Non_Interlock Mask Settings pp: X, q: Y, rr: W, ss: H |
| X |  | GridOn | 8x 01 04 7C 02 FF | Grid Display ON/OFF |
| X |  | GridOff | 8x 01 04 7C 03 FF | Grid/Center Line Display Off |
| X |  | CenterLineOn | 8x 01 04 7C 04 FF | Center Line Display On |
| X |  | Off | 8x 01 04 17 00 FF | Camera Control Enable/Disable |
| X | CAM_KeyLock | On | 8x 01 04 17 02 FF |  |
| OK | CAM_IDWrite | – | 8x 01 04 22 0p 0q 0r 0s FF | pqrs: Camera ID (=0000 to FFFF) |
| OK | CAM_MemSave | Write | 8x 01 04 23 0m 0p 0q 0r 0s FF | m: Address (=0 to 7) pqrs: Data (0000 to FFFF) |
| SHANY V | Command Set | Command | Command Packet | Comments |
| X |  | Internal Mode | 8x 01 04 55 00 FF | Internal Sync |
| X |  | Line Lock Mode | 8x 01 04 55 01 FF | V-Phase Adjustment |
| X | CAM_ExtLock | Frequency Lock Mode | 8x 01 04 55 02 FF | External Sync |
| X |  | Stop | 8x 01 04 05 00 FF | – |
| X |  | Up | 8x 01 04 05 02 FF |  |
| X |  | Down | 8x 01 04 05 03 FF |  |
| X |  | Up (Step) | 8x 01 04 05 2p FF | p=step (1-7) |
| X | CAM_VPhase | Down (Step) | 8x 01 04 05 3p FF |  |
| X |  | Reset | 8x 01 04 05 40 FF | Restore Factory Settings |
| X |  | Direct | 8x 01 04 45 0p 0q 0r 0s FF | pqrs: V-Phase (0000 to) (NTSC: 0000 to 20Chex, PAL: 0000 to 270hex) |
| X |  | On | 8x 01 04 6B 02 FF | Alarm ON/OFF |
| X |  | Off | 8x 01 04 6B 03 FF |  |
| X | CAM_Alarm | SetMode | 8x 01 04 6C pp FF | pp: Mode setting 00 Focus change detection (reference value is not updated) 01 Focus change detection (reference value is updated) 02 AE change detection (reference value is not updated) 03 AE change detection (reference value is updated) :0C Day/Night judgement |
| X |  | SetDayNighLev el | 8x 01 04 6D 0p 0p 0p 0q 0q 0q FF | ppp: Day judgement level setting qqq: Night judgement level setting |
| X |  |  | y0 07 04 6B 01 FF | Detection level “ Low” → “High” |
| X |  | Alarm(Reply) | y0 07 04 6B 00 FF | Detection level “ High” → “Low” |
| X |  | On | 8x 01 04 1B 02 FF | Motion Detection On/Off |
| X |  | Off | 8x 01 04 1B 03 FF |  |
| X |  | Function Set | 8x 01 04 1C 0m 0n 0p 0q 0r 0s FF | m: Display mode n: Detection Frame Set (0 to F) pq: Threshold Level (00 to FF) rs: Interval Time set (00 to FF) |
| X | CAM_MD | Window Set | 8x 01 04 1D 0m 0p 0q 0r 0s FF | m: Select Detection Frame (0, 1, 2, 3) p: Start Horizontal Position (00 to 0B) q: Start Vertical Position (00 to 07) r: Stop Horizontal Position (01 to 0C) s: Stop Vertical Position (01 to 08) |
| X |  | Alarm (Reply) | y0 07 04 1B 0p FF | p: Detection Frame Number |
| X |  | On | 8x 01 04 69 02 FF | Zoom Position data Continuous Output On/Off |
| X |  | Off | 8x 01 04 69 03 FF |  |
| X | CAM_Continuous ZoomPosReply | (Reply) | y0 07 04 69 0p 0p 0q 0q 0q 0q FF | pp: D-Zoom Position * 00: When Zoom Mode is Combine qqqq: Zoom Position |
| X | CAM_ ReplyIntervalTimeSet | – | 8x 01 04 6A 00 00 0p 0p FF | pp: Interval Time [Vertical timing] |
| X | CAM_RegisterValue | – | 8x 01 04 24 mm 0p 0p FF | mm: Register No. (=00-7F) pp: Register Value (=00-7F) |
| SHANY V | Command Set | Command | Command Packet | Comments |
| X | CAM_ColorEnhance | Parameter Set | 8x 01 04 20 mm nn pp qq rr ss tt uu FF | mm: Threshold level nn: Hysteresis width pp: Fixed color Y of high-intensity side qq: Fixed color Cr of high-intensity side rr: Fixed color Cb of high-intensity side ss: Fixed color Y of low-intensity side tt: Fixed color Cr of low-intensity side uu: Fixed color Cb of low-intensity side * Set 00h to 7Fh for each parameter. |
| X |  | On | 8x 01 04 50 02 FF | Color Enhancement ON/OFF |
| X |  | Off | 8x 01 04 50 03 FF |  |

查询命令：

| SHANY V | Inqulry Command | Command Packet | Inqulry Packet | Comments |
| --- | --- | --- | --- | --- |
| OK |  |  | y0 50 02 FF | On |
| OK | CAM_PowerInq | 8x 09 04 00 FF | y0 50 03 FF | Off |
| OK | CAM_ZoomPosInq | 8x 09 04 47 FF | y0 50 0p 0q 0r 0s FF | pqrs: Zoom Position |
| X |  |  | y0 50 02 FF | D-Zoom On |
| X | CAM_DZoomModeInq | 8x 09 04 06 FF | y0 50 03 FF | D-Zoom Off |
| X |  |  | y0 50 00 FF | Combine Mode |
| X | CAM_DZoomC/SModeInq | 8x 09 04 36 FF | y0 50 01 FF | Separate Mode |
| X | CAM_DZoomPosInq | 8x 09 04 46 FF | y0 50 00 00 0p 0q FF | pq: D-Zoom Position |
| OK |  |  | y0 50 02 FF | Auto Focus |
| OK | CAM_FocusModeInq | 8x 09 04 38 FF | y0 50 03 FF | Manual Focus |
| OK | CAM_FocusPosInq | 8x 09 04 48 FF | y0 50 0p 0q 0r 0s FF | pqrs: Focus Position |
| OK | CAM_FocusNearLimitInq | 8x 09 04 28 FF | y0 50 0p 0q 0r 0s FF | pqrs: Focus Near Limit Position |
| X |  |  | y0 50 02 FF | AF Sensitivity Normal |
| X | CAM_AFSensitivityInq | 8x 09 04 58 FF | y0 50 03 FF | AF Sensitivity Low |
| X |  |  | y0 50 00 FF | Normal AF |
| X | CAM_AFModeInq | 8x 09 04 57 FF | y0 50 01 FF | Interval AF |
| X |  |  | y0 50 02 FF | Zoom Trigger AF |
| X | CAM_AFTimeSettingInq | 8x 09 04 27 FF | y0 50 0p 0q 0r 0s FF | pq: Movement Time, rs: Interval |
| X |  |  | y0 50 00 FF | Standard |
| X | CAM_IRCorrectionInq | 8x 09 04 11 FF | y0 50 01 FF | IR Light |
| OK |  |  | y0 50 00 FF | Auto |
| OK |  |  | y0 50 01 FF | In Door |
| OK |  |  | y0 50 02 FF | Out Door |
| OK |  |  | y0 50 03 FF | One Push WB |
| OK |  |  | y0 50 04 FF | ATW |
| OK | CAM_WBModeInq | 8x 09 04 35 FF | y0 50 05 FF | Manual |
| X |  |  | y0 50 06 FF | Outdoor Auto |
| X |  |  | y0 50 07 FF | Sodium Lamp Auto |
| X |  |  | y0 50 08 FF | Sodium Lamp |
| OK |  |  | y0 50 09 FF | CRS |
| X | CAM_RGainInq | 8x 09 04 43 FF | y0 50 00 00 0p 0q FF | pq: R Gain |
| SHANY V | Inqulry Command | Command Packet | Inqulry Packet | Comments |
| X | CAM_BGainInq | 8x 09 04 44 FF | y0 50 00 00 0p 0q FF | pq: B Gain |
| OK |  |  | y0 50 00 FF | Full Auto |
| OK |  |  | y0 50 03 FF | Manual |
| OK | CAM_AEModeInq | 8x 09 04 39 FF | y0 50 0A FF | Shutter Priority |
| X |  |  | y0 50 0B FF | Iris Priority |
| X |  |  | y0 50 0D FF | Bright |
| X |  |  | y0 50 02 FF | Auto |
| X | CAM_SlowShutterModeInq | 8x 09 04 5A FF | y0 50 03 FF | Manual |
| OK | CAM_ShutterPosInq | 8x 09 04 4A FF | y0 50 00 00 0p 0q FF | pq: Shutter Position |
| X | CAM_IrisPosInq | 8x 09 04 4B FF | y0 50 00 00 0p 0q FF | pq: Iris Position |
| OK | CAM_GainPosInq | 8x 09 04 4C FF | y0 50 00 00 0p 0q FF | pq: Gain Position |
| X | CAM_GainLimitInq | 8x 09 04 2C FF | y0 50 0q FF | p: Gain Limit |
| OK | CAM_BrightPosInq | 8x 09 04 4D FF | y0 50 00 00 0p 0q FF | pq: Bright Position |
| X |  |  | y0 50 02 FF | On |
| X | CAM_ExpCompModeInq | 8x 09 04 3E FF | y0 50 03 FF | Off |
| OK | CAM_ExpCompPosInq | 8x 09 04 4E FF | y0 50 00 00 0p 0q FF | pq: ExpComp Position |
| X |  |  | y0 50 02 FF | On |
| X | CAM_BacklightModeInq | 8x 09 04 33 FF | y0 50 03 FF | Off |
| X |  |  | y0 50 02 FF | On |
| X | CAM_SpotAEModeInq | 8x 09 04 59 FF | y0 50 03 FF | Off |
| X | CAM_SpotAEPosInq | 8x 09 04 29 FF | y0 50 0p 0q 0r 0s FF | pq: X position, rs: Y position |
| X | CAM_AE_ResponseInq | 8x 09 04 5D FF | y0 50 pp FF | pp: 01 to 20 (hex) |
| OK |  |  | y0 50 02 FF | On Wide-D |
| OK |  |  | y0 50 03 FF | Off |
| X | CAM_WDModeInq | 8x 09 04 3D FF | y0 50 00 FF | AutoOnOff |
| X |  |  | y0 50 01 FF | On (RatioFix) |
| X |  |  | y0 50 04 FF | On (Dver operation) |
| X | CAM_WDParameterInq | 8x 09 04 2D FF | y0 50 0p 0q 0r 0s 0t 0u 00 00 FF | p: Screen display q: Detection sensitivity r: Blocked-up shadow correction level s: Blown-out highlight correction level tu: Exposure ratio of short exposure |
| X |  |  | y0 50 02 FF | On |
| X | CAM_WDAlarmReplyInq | 8x 09 04 3B FF | y0 50 03 FF | Off |
| OK | CAM_ApertureInq | 8x 09 04 42 FF | y0 50 00 00 0p 0q FF | pq: Aperture Gain |
| X |  |  | y0 50 02 FF | On Hi-Resolution |
| X | CAM_HRModeInq | 8x 09 04 52 FF | y0 50 03 FF | Off |
| X | CAM_NRInq | 8x 09 04 53 FF | y0 50 0p FF | p: NR level |
| OK | CAM_LR_ReverseModeIn |  | y0 50 02 FF | On |
| OK | q | 8x 09 04 61 FF | y0 50 03 FF | Off |
| X |  |  | y0 50 02 FF | On |
| X | CAM_FreezeModeInq | 8x 09 04 62 FF | y0 50 03 FF | Off |
| OK |  |  | y0 50 00 FF | Off |
| OK | CAM_PictureEffectModeIn | 8x 09 04 63 FF | y0 50 02 FF | Neg.Art |
| OK | q |  | y0 50 04 FF | B&W |
| X |  |  | y0 50 02 FF | On |
| X | CAM_PictureFlipModeInq | 8x 09 04 66 FF | y0 50 03 FF | Off |
| OK |  |  | y0 50 02 FF | On |
| OK | CAM_ICRModeInq | 8x 09 04 01 FF | y0 50 03 FF | Off |
| OK |  |  | y0 50 02 FF | On |
| SHANY V | Inqulry Command | Command Packet | Inqulry Packet | Comments |
| OK | CAM_AutoICRModeInq | 8x 09 04 51 FF | y0 50 03 FF | Off |
| X | CAM_AutoICRThresholdIn q | 8x 09 04 21 FF | y0 50 00 00 0p 0q FF | pq: ICR ON → OFF Threshold Level |
| X | CAM_AutoICRAlarmReply |  | y0 50 02 FF | On |
| X | Inq | 8x 09 04 31 FF | y0 50 03 FF | Off |
| OK |  | 8x 09 04 15 FF | y0 50 02 FF | On |
| OK | CAM_DisplayModeInq | (8x 09 06 06 FF) | y0 50 03 FF | Off |
| OK |  |  | y0 50 02 FF | On |
| OK | CAM_MuteModeInq | 8x 09 04 75 FF | y0 50 03 FF | Off |
| X | CAM_PrivacyDisplayInq | 8x 09 04 77 FF | y0 50 pp pp pp pp FF | pp pp pp pp: Mask Display (0:OFF, 1:ON) |
| X | CAM_PrivacyPanTiltInq | 8x 09 04 79 FF | y0 50 0p 0p 0p 0q 0q 0q FF | ppp: Pan qqq: Tilt |
| X | CAM_PrivacyPTZInq | 8x 09 04 7B mm FF | y0 50 0p 0p 0p 0q 0q 0q 0r 0r 0r 0r FF | mm: Mask Settings ppp: Pan qqq: Tilt rrr: Zoom |
| X | CAM_PrivacyMonitorInq | 8x 09 04 6F FF | y0 50 pp pp pp pp FF | pp pp pp pp: Mask is displayed now. |
| X |  |  | y0 50 00 FF | Off |
| X | CAM_KeyLockInq | 8x 09 04 17 FF | y0 50 02 FF | On |
| OK | CAM_IDInq | 8x 09 04 22 FF | y0 50 0p 0q 0r 0s FF | pqrs: Camera ID |
| OK | CAM_MemSaveInq | 8x 09 04 23 0X FF | y0 50 0p 0p 0q 0q FF | X: 00 to 07 (Address) ppqq: 0x0000 to 0xFFFF (Data) |
| OK |  |  | y0 50 00 FF | Internal Sync |
| X | CAM_ExtLockInq | 8x 09 04 55 FF | y0 50 01 FF | V-Phase Adjustment |
| X |  |  | y0 50 02 FF | External Sync |
| X | CAM_VPhasePosInq | 8x 09 04 45 FF | y0 50 0p 0q 0r 0s FF | pqrs: V-Phase Position |
| OK | CAM_VersionInq | 8x 09 00 02 FF | y0 50 00 20 mn pq rs tu vw FF | mnpq: Model Code (04xx) rstu: ROM version vw: Socket Number (=02) |
| X |  |  | y0 50 02 FF | On |
| X | CAM_AlarmInq | 8x 09 04 6B FF | y0 50 03 FF | Off |
| X | CAM_AlarmModeInq | 8x 09 04 6C FF | y0 50 pp FF | pp: Alarm Mode |
| X | CAM_AlarmDayNightLe velInq | 8x 09 04 6D FF | y0 50 0p 0p 0p 0q 0q 0q 0r 0r 0r FF | ppp: Day judgement level setting qqq: Night judgement level setting rrr: Current Automatic Exposure level setting |
| X | CAM_AlarmDetectLevelI |  | y0 50 01 FF | Detection level “High” |
| X | nq | 8x 09 04 6E FF | y0 50 00 FF | Detection level “Low” |
| X |  |  | y0 50 02 FF | On |
| X | CAM_MDModeInq | 8x 09 04 1B FF | y0 50 03 FF | Off |
| X | CAM_MDFunctionInq | 8x 09 04 1C FF | y0 50 0m 0n 0p 0q FF | m: Display mode n: Detection Frame Set (0 to F) pq: Threshold Level (0 to FF) rs: Interval Time set (0 to FF) |
| X | CAM_MDWindowInq | 8x 09 04 1D 0m FF | y0 50 0p 0q 0r 0s FF | m: Select Detection Frame (0, 1, 2, 3) p: Start Horizontal Position (00 to 0B) q: Start Vertical Position (00 to 07) r: Stop Horizontal Position (01 to 0C) s: Stop Vertical Position (01 to 08) |
| X | CAM_ContinuousZoomPos |  | y0 50 02 FF | On |
| SHANY V | Inqulry Command | Command Packet | Inqulry Packet | Comments |
| X | ReplyModeInq | 8x 09 04 69 FF | y0 50 03 FF | Off |
| X | CAM_ReplyIntervalTime Inq | 8x 09 04 6A FF | y0 50 00 00 0p 0p FF | pp: Interval Time |
| X | CAM_RegisterValueInq | 8x 09 04 24 mm FF | y0 50 0p 0p ff | mm: Register No. (00 to 7F) pp: Register Value (00 to FF) |
| X | CAM_ColorEnhanceInq | 8x 09 04 20 FF | y0 50 mm nn pp qq rr ss tt uu FF | mm: Threshold level nn: Hysteresis width pp: Fixed color Y of high-intensity qq: Fixed color Cr of high-intensity rr: Fixed color Cb of high-intensity ss: Fixed color Y of low-intensity tt: Fixed color Cr of low-intensity uu: Fixed color Cb of low-intensity |
| X |  |  | y0 50 02 FF | On |
| X |  | 8x 09 04 50 FF | y0 50 03 FF | Off |
| X | CAM_TempInq | 8x 09 04 68 FF | y0 50 00 00 0p 0p FF | pq: Temperature * Lens temperature |

- VISCA Command Setting Values

- Register Setting:

| CAM_RegisterValue |  | 8x 01 04 24 mm 0p 0q FF | mm: Register NO pp: Register Value |
| --- | --- | --- | --- |

|  | Register No. | Value |  |  |
| --- | --- | --- | --- | --- |
| VISCA Baud Rate | 00 | 00 (default value) | 9600 bps |  |
|  |  | 01 | 19200 bps |  |
|  |  | 02 | 38400 bps |  |
| Monitoring Mode | 72 | 06 | 1080p@30 | 122/222/136/236支持 |
|  |  | 08 | 1080p@25 | 122/222/136/236支持 |
|  |  | 0E | 720P@30 | 122/222/136/236支持 |
|  |  | 11 | 720P@25 | 122/222/136/236支持 |
|  |  | 0C | 720P@50 | 136/236/138/238支持 |
|  |  | 09 | 720P@60 | 136/236/138/238支持 |
|  |  | 20 | 1080P@50 | 136/236支持 |
|  |  | 21 | 1080P@60 | 136/236支持 |

Exposure Control

| Shutter Speed （范围0x00 ~ 0x0f） | 数值 |  |
| --- | --- | --- |
|  | 0F | 1/10000 |
|  | 0E | 1/6000 |
|  | 0D | 1/3500 |
|  | 0C | 1/2500 |
|  | 0B | 1/1750 |
|  | 0A | 1/1000 |
|  | 09 | 1/600 |
|  | 08 | 1/425 |
|  | 07 | 1/300 |
|  | 06 | 1/215 |
|  | 05 | 1/150 |
|  | 04 | 1/120 |
|  | 03 | 1/100 |
|  | 02 | 1/75 |
|  | 01 | 1/50 |
|  | 00 | 1/25 |

| Gain （范围0x00 ~ 0x0f） | 0F | +30 dB |
| --- | --- | --- |
|  | 0E | +28 dB |
|  | 0D | +26 dB |
|  | 0C | +24 dB |
|  | 0B | +22 dB |
|  | 0A | +20 dB |
|  | 09 | +18 dB |
|  | 08 | +16 dB |
|  | 07 | +14 dB |
|  | 06 | +12 dB |
|  | 05 | +10 dB |
|  | 04 | +8 dB |
|  | 03 | +6 dB |
|  | 02 | +4 dB |
|  | 01 | +2 dB |
|  | 00 | 0 dB |

Zoom Ratio Optical Zoom

×10 Lens Position Data

| Zoom Ratio | Optical Zoom |
| --- | --- |
|  | Positon Data |
| x1 | 0000 |
| x2 | 060C |
| x3 | 0F96 |
| x4 | 1A11 |
| x5 | 245E |
| x6 | 2CE1 |
| x7 | 3507 |
| x8 | 3B4D |
| x9 | 3E7B |
| x10 | 4000 |

Zoom Ratio Optical Zoom

×18 Lens Position Data

| Zoom Ratio | Optical Zoom |
| --- | --- |
|  | Positon Data |
| x1 | 0000 |
| x2 | 02FD |
| x3 | 07F7 |
| x4 | 0DC5 |
| x5 | 129B |
| x6 | 1771 |
| x7 | 1D3E |
| x8 | 221B |
| x9 | 26E3 |
| x10 | 2B86 |
| x11 | 30C9 |
| x12 | 34B6 |
| x13 | 382E |
| x14 | 3B73 |
| x15 | 3D45 |
| x16 | 3E69 |
| x17 | 3F35 |
| x18 | 4000 |
| Zoom Ratio | Optical Zoom |
|  | Positon Data |
| x1 | 0000 |
| x2 | 01F5 |
| x3 | 04E6 |
| x4 | 0853 |
| x5 | 0BC1 |
| x6 | 0EB2 |
| x7 | 121F |
| x8 | 158D |
| x9 | 187E |
| x10 | 1BEB |
| x11 | 1F59 |
| x12 | 224A |
| x13 | 259E |
| x14 | 285C |
| x15 | 2AB6 |
| x16 | 2D75 |
| x17 | 3033 |
| x18 | 325B |
| x19 | 34C1 |
| x20 | 3702 |
| x21 | 38C5 |
| x22 | 3A7C |
| x23 | 3BCF |
| x24 | 3CB1 |
| x25 | 3D9F |
| x26 | 3E7A |
| x27 | 3F17 |
| x28 | 4000 |

- Title setting

| Line number | 00 |  |
| --- | --- | --- |
| H-position | 00 to 17 |  |
| Blink | 00: Dose not blink |  |
|  | 01: Blinks |  |
|  | 00 | White |
|  | 01 | Yellow |
|  | 02 | Violet |
| Color | 03 | Red |
|  | 04 | Cyan |
|  | 05 | Green |
|  | 06 | Blue |

| 00 | 01 | 02 03 04 | 05 |
| --- | --- | --- | --- |
| A | B | C D E | F |
| 06 | 07 | 08 09 0a | 0b |
| G | H | I J K | L |
| 0c | 0d | 0e 0f 10 | 11 |
| M | N | O P Q | R |
| 12 | 13 | 14 15 16 | 17 |
| S | T | U V W | X |
| 18 | 19 | 1a 1b 1c | 1d |
| Y | Z | & ? | ! |
| 1e | 1f | 20 21 22 | 23 |
| 1 | 2 | 3 4 5 | 6 |
| 24 | 25 | 26 27 49 4a | 4b |
| 7 | 8 | 9 0 “ : | ‘ |
| 4c | 4d | 4e 4f 50 51 52 | 53 |
| . | , | / - ( ) * | @ |

编号50H～52H为扩展字符