22X（1/4〞）一体机机芯式样书

#### Model

FTZ22P48-T2		FTZ22N48-T2

#### Production

#### Specification

| Zoom Camera |  |
| --- | --- |
| Model | FTZ22P48-T2 |
| Resolution | 480TVL |
| Communication mode | RS232 |
| Signal system | PAL/NTSC |
| CCD | 1/4" CMOS |
| Effective pixels | PAL:510(H)×576(V) / NTSC:510(H)×487(V) |
| Day & Night | Colour/ B&W |
| Mini.Illumination | 0.01Lux(infra LED on) |
| Sync System | Inter-sync |
| Video output | CVBS Output (1Vp-p75 Ω) |
| SNR | ≥48dB |
| Lens |  |
| FNO./Focal Length | f=4mm～88mm |
| IRCUT | IRCUT |
| AUTO IRIS | —— |
| M.O.D | 0.2m（W）～1m（T） |
| Functions |  |
| OSD Display | Support |
| Lens Initialization | Auto |
| Exposure Mode | Auto |
| AWB | Auto |
| Focus Mode | Auto/Manual |
| Picture Effects | Auto/Color/B&W |
| Backlight Compensator | Support |
| Contrast | Adjust |
| Saturation | Adjust |
| Mirror Image | Support(LR) |
| Digital Zoom | —— |
| Freeze | —— |
| Rollovers | —— |
| DNR | 2D-DNR |
| General Specifications |  |
| Size | 47（W）* 57（H）* 83（L） mm |
| Working Temp./Hygro. | ﹣10℃～50℃，10％RH～60％RH |
| Storage Temp./Hygro. | ﹣20℃～60℃，10％RH～80％RH |
| Power Voltage | DC 12V±10% |
| Power consumption | 3.0W MAX |

#### Pin assignment

The Connect Pins Functions Table: the order of 1 to 9 pin location and the following table.

| Pin NO. | Name | Specification |
| --- | --- | --- |
|  | RxD | CMOS 3.3V (low: max 0.8V, high: min 2.8V) Read Date |
|  | TxD | CMOS 3.3V (low: max 0.8V, high: min 2.8V) Send Date |
|  | GND |  |
|  | DC IN | 12V±3V |
|  | GND |  |
|  | CVBS OUT | 1.0V±0.2V |
|  | GND |  |
|  | NC |  |
|  | GND |  |

Command List

Command List

## （注明：命令中红色标记项为可选项，根据产品型号对应功能。）

#### Function Command List

| Command Set | Command | Command Packet | Comments |
| --- | --- | --- | --- |
| Network Change | Broadcast | 88 30 0x FF |  |
| IF_Clear | Broadcast | 88 01 00 01 FF |  |
| CAM_Power | On | 8x 01 04 00 02 FF | Power ON/OFF |
|  | Off | 8x 01 04 00 03 FF |  |
| CAM_Zoom | Stop | 8x 01 04 07 00 FF | Auto Focus after Zoom change(FT331 Only) |
|  | Tele(Standard) | 8x 01 04 07 02 FF |  |
|  | Wide(standard) | 8x 01 04 07 03 FF |  |
|  | Tide(Variable) | 8x 01 04 07 2p FF | p=0(low) to 7(High) |
|  | Wide(Variable) | 8x 01 04 07 3p FF |  |
|  | Direct | 8x 01 04 47 0p 0q 0r 0s FF | pqrs: Zoom Position, Must set Focus Direct command in Manual Focus Mode. (FT331 Only ) |
| CAM_DZoom | On | 8x 01 04 06 02 FF | Digital zoom ON/OFF |
|  | Off | 8x 01 04 06 03 FF |  |
|  | Combine Mode | 8x 01 04 36 00 FF | Optical /Digital Zoom Combined |
|  | Separate Mode | 8x 01 04 36 01 FF | Optical /Digital Zoom Separate |
|  | Stop | 8x 01 04 06 00 FF |  |
|  | Tele (Variable) | 8x 01 04 06 2p FF | p=0 (Low) to 7 (High) |
|  | Wide (Variable) | 8x 01 04 06 3p FF |  |
|  | x1/Max | 8x 01 04 06 10 FF | x1/Max Magnification Switchover |
|  | Direct | 8x 01 04 06 00 00 0p 0q FF | pq: D-Zoom Position |
| CAM_Focus | Stop | 8x 01 04 08 00 FF | Munual Focus Mode |
|  | Far(Standard) | 8x 01 04 08 02 FF |  |
|  | Near(Standard) | 8x 01 04 08 03 FF |  |
|  | Far(Variable) | 8x 01 04 08 2p FF | p=0(low) to 7(High) |
|  | Near(Variable) | 8x 01 04 08 3p FF |  |
|  | Direct | 8x 01 04 48 0p 0q 0r 0s FF | pqrs: Focus Position，Must set Zoom Direct command in Manual Focus Mode. (FT331 Only ) |
|  | Auto Focus | 8x 01 04 38 02 FF | AF ON/OFF |
|  | Manual Focus | 8x 01 04 38 03 FF |  |
|  | Auto/ Manual | 8x 01 04 38 10 FF |  |
|  | One Push Trigger | 8x 01 04 18 01 FF |  |
|  | Infinity | 8x 01 04 18 02 FF |  |
|  | Near Limit | 8x 01 04 28 0p 0q 0s FF | pqrs: Focus Near Limit Position |
| AF Sensitivity | Normal | 8x 01 04 58 02 FF | AF Sensitivity High/Low |
|  | Low | 8x 01 04 58 03 FF |  |
| CAM_AF Mode | Normal AF | 8x 01 04 57 00 FF | AF Movement Mode |
|  | Interval AF | 8x 01 04 57 01 FF |  |
|  | ZoomTriggerAF | 8x 01 04 57 02 FF |  |
|  | Active/Interval Time | 8x 01 04 27 0p 0q 0r 0s FF | pq:Movement Time, rs:Interval |
| CAM_Zoom Focus | Direct | 8x 01 04 47 0p 0q 0r 0s 0t 0u 0v 0w FF | pqrs: Zoom Position tuvw: Focus Position |
| CAM_Initialize | Lens | 8x 01 04 19 01 FF | Lens Initialization start |
| CAM_WB | Auto | 8x 01 04 35 00 FF | Normal Auto |
|  | Indoor | 8x 01 04 35 01 FF | Indoor mode |
|  | Outdoor | 8x 01 04 35 02 FF | Outdoor mode |
|  | ATW | 8x 01 04 35 04 FF | Auto |
|  | Manual | 8x 01 04 35 05 FF | Manual Control mode |
| CAM_R Gain | Reset | 8x 01 04 03 00 FF | Manual Control of R Gain |
|  | Up | 8x 01 04 03 02 FF |  |
|  | Down | 8x 01 04 03 03 FF |  |
|  | Direct | 8x 01 04 43 00 00 0p 0q FF | pq: R Gain |
| CAM_B Gain | Reset | 8x 01 04 04 00 FF | Manual Control of B Gain |
|  | Up | 8x 01 04 04 02 FF |  |
|  | Down | 8x 01 04 04 03 FF |  |
|  | Direct | 8x 01 04 44 00 00 0p 0q FF | pq: B Gain |
| CAM_AE | Full Auto | 8x 01 04 39 00 FF | Automatic Exposure mode |
|  | Manual | 8x 01 04 39 03 FF | Manual Control mode |
| CAM_Gain | Reset | 8x 01 04 0C 00 FF | Gain Setting |
|  | Up | 8x 01 04 0C 02 FF |  |
|  | Down | 8x 01 04 0C 03 FF |  |
|  | Direct | 8x 01 04 4C 00 00 0p 0q FF | pq: Gain Position |
| CAM_Backlight | On | 8x 01 04 33 02 FF | Back Light Compensation ON/OFF |
|  | Off | 8x 01 04 33 03 FF |  |
| CAM_LR_ Reverse | On | 8x 01 04 61 02 FF | Mirror Image ON/OFF |
|  | Off | 8x 01 04 61 03 FF |  |
| CAM_Freeze | On | 8x 01 04 62 02 FF | Still Image ON/OFF |
|  | Off | 8x 01 04 62 03 FF |  |
| CAM_Picture Effect | Off | 8x 01 04 63 00 FF | Picture Effect Setting |
|  | Neg.Art | 8x 01 04 63 02 FF |  |
|  | B&W | 8x 01 04 63 04 FF |  |
| CAM_PictureFlip | On | 8x 01 04 66 02 FF | Picture flip ON/OFF |
|  | Off | 8x 01 04 66 03 FF |  |
| CAM_ICR | On | 8x 01 04 01 02 FF | Infrared Mode ON/OFF |
|  | Off | 8x 01 04 01 03 FF |  |
| CAM_AutoICR | On | 8x 01 04 01 02 FF | Auto Infrared Mode ON/OFF |
|  | Off | 8x 01 04 01 03 FF |  |
| CAM_Memory | Reset | 8x 01 04 3F 00 0p FF | p: Memory Number (=0 to 5) |
|  | Set | 8x 01 04 3F 01 0p FF |  |
|  | Recall | 8x 01 04 3F 02 0p FF |  |
| CAM_CUSTOM | Reset | 8x 01 04 3F 00 7F FF | Starts in this mode at Power ON. |
|  | Set | 8x 01 04 3F 01 7F FF |  |
|  | Recall | 8x 01 04 3F 02 7F FF |  |
| CAM_MemSave | Write | 8x 01 04 23 0X 0p 0p 0q 0q FF | X: 00 to 07 (Address) Total 16 Byte ppqq: 0x0000 to 0xFFFF (Data) |
| CAM_Display | On | 8x 01 04 15 02 FF | Display: ON/OFF |
|  | Off | 8x 01 04 15 03 FF |  |
|  | On/Off | 8x 01 04 15 10 FF |  |
| CAM_Title | Title Set1 | 8x 01 04 73 00 mm nn pp qq 00 00 00 00 00 00 FF | mm:Vposition, nn:Hposition, pp:Color, qq:Blink |
|  | Title Set2 | 8x 01 04 73 01 mm nn pp qq rr ss tt uu vv ww FF | mnpqrstuvw:Setting of Display Characters(1st to10st Character ) |
|  | Title Set3 | 8x 01 04 73 02 mm nn pp qq rr ss tt uu vv ww FF | mnpqrstuvw: Setting of Display Characters(11st to 20st Character) |
|  | Title Clear | 8x 01 04 74 00 FF | Title Setting Clear |
|  | On | 8x 01 04 74 02 FF | Title Display ON/OFF |
|  | Off | 8x 01 04 74 03 FF |  |
| CAM_Mute | On | 8x 01 04 75 02 FF | Mute ON/OFF |
|  | Off | 8x 01 04 75 03 FF |  |
|  | On/Off | 8x 01 04 75 10 FF |  |
| CAM_ID Write |  | 8x 01 04 22 0p 0q 0r 0s FF | pqrs: Camera ID (=0000 to FFFF) |

#### Inquiry Command List

| Inquiry Command | Command Packet | Inquiry Packet | Comments |
| --- | --- | --- | --- |
| CAM_PowerInq | 8x 09 04 00 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_ZoomPosInq | 8x 09 04 47 FF | y0 50 0p 0q 0r 0s FF | pqrs:ZoomPosition |
| CAM_DZoomModeInq | 8x 09 04 06 FF | y0 50 02 FF | D-Zoom On |
|  |  | y0 50 03 FF | D-Zoom Off |
| CAM_DZoomC/SModeInq | 8x 09 04 36 FF | y0 50 00 FF | Combine Mode |
|  |  | y0 50 01 FF | Separate Mode |
| CAM_DZoomPosInq | 8x 09 04 46 FF | y0 50 00 00 0p 0q FF | Pq: D-Zoom Position |
| CAM_FocusModeInq | 8x 09 04 38 FF | y0 50 02 FF | Auto Focus |
|  |  | y0 50 03 FF | Manual |
| CAM_FocusPosInq | 8x 09 04 48 FF | y0 50 0p 0q 0r 0s FF | pqrs:FocusPosition |
| CAM_FocusNear LimitInq | 8x 09 04 28 FF | y0 50 0p 0q 0r 0s FF | pqrs: Focus Near Limit Position |
| CAM_AFModeInq | 8x 09 04 57 FF | y0 50 00 FF | Normal AF |
|  |  | y0 50 01 FF | Interval AF |
|  |  | y0 50 02 FF | Zoom Trigger AF |
| CAM_AFTime SettingInq | 8x 09 04 27 FF | y0 50 0p 0q 0r 0s FF | pq: Movement Time, rs: Interval |
| CAM_WBModeInq | 8x 09 04 35 FF | y0 50 00 FF | Auto |
|  |  | y0 50 01 FF | In Door |
|  |  | y0 50 02 FF | Out Door |
|  |  | y0 50 05 FF | Manual |
| CAM_RGainInq | 8x 09 04 43 FF | y0 50 00 00 0p 0q FF | pq: R Gain |
| CAM_BGainInq | 8x 09 04 44 FF | y0 50 00 00 0p 0q FF | pq: B Gain |
| CAM_AEModeInq | 8x 09 04 39 FF | y0 50 00 FF | Full Auto |
|  |  | y0 50 03 FF | Manual |
|  |  | y0 50 0B FF | Iris Priority |
| CAM_GainPosInq | 8x 09 04 4C FF | y0 50 00 00 0p 0q FF | pq: Gain Priority |
| CAM_BacklightModeInq | 8x 09 04 33 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
|  |  | y0 50 03 FF | Off |
| CAM_LR_ReverseModeInq | 8x 09 04 61 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_FreezeModeInq | 8x 09 04 62 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_PictureEffectModeInq | 8x 09 04 63 FF | y0 50 00 FF | Off |
|  |  | y0 50 02 FF | Neg.Art |
|  |  | y0 50 04 FF | B&W |
| CAM_PictureFlipModeInq | 8x 09 04 66 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_ICRModeInq | 8x 09 04 01 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_AutoICRModeInq | 8x 09 04 51 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_MemoryInq | 8x 09 04 3F FF | y0 50 pp FF | pp: Last Recall Memory No. |
| CAM_MemSaveInq | 8x 09 04 23 0x FF | y0 50 0p 0p 0q 0q FF | X:00～07(Address) ppqq: 0x0000 to 0xFFFF (Data) |
| CAM_DisplayModeInq | 8x 09 04 15 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_MuteModeInq | 8x 09 04 75 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_TitleDisplay ModeInq | 8x 09 04 74 FF | y0 50 02 FF | On |
|  |  | y0 50 03 FF | Off |
| CAM_IDInq | 8x 09 04 22 FF | y0 50 0p 0q 0r 0s FF | pqrs: Camera ID |

菲特机芯查询命令：

| 自检查询机器型号 | 查询命令 | 返回码 | 注释 |
| --- | --- | --- | --- |
| pp qq | 81 09 04 B2 FF | 90 50 B2 pp qq FF | pp qq：与型号对应的4位数字 |

备注：菲特机芯同样兼容SONY FCB-EX480查询命令，请注意查询顺序！！！

菲特机芯型号与返回码对应表如下：

| 机芯型号 | 查询命令 | 返回码 | 注释 |
| --- | --- | --- | --- |
| FTZ22P80-T2 | 81 09 04 B2 FF | 90 50 B2 73 22 FF | pp qq ：73 22（22x CMOS 800TVL ICR） |
| FTZ22P48-I2 | 81 09 04 B2 FF | 90 50 B2 73 21 FF | pp qq ：73 21（22x Effio 480TVL NO ICR） |
| FTZ22P48-T2 | 81 09 04 B2 FF | 90 50 B2 73 23 FF | pp qq ：73 23（22x Effio 480TVL ICR） |
| FTZ26P48-I2 | 81 09 04 B2 FF | 90 50 B2 73 26 FF | pp qq ：73 26（26x Effio 480TVL NO ICR） |
| FTZ26P65-T2A | 81 09 04 B2 FF | 90 50 B2 73 26 FF | pp qq ：73 26（26x Effio 480TVL ICR） |
| FTZ18P65-T2 | 81 09 04 B2 FF | 90 50 B2 71 53 FF | pp qq ：71 53（18x Effio 650TVL ICR） |
| FTZ18N65-T2 | 81 09 04 B2 FF | 90 50 B2 71 53 FF | pp qq ：71 53（18x Effio 650TVL ICR） |
| FTZ10P65-T2 | 81 09 04 B2 FF | 90 50 B2 71 10 FF | pp qq ：71 10（10x Effio 650TVL ICR） |
| FTZ10P65-T2E | 81 09 04 B2 FF | 90 50 B2 71 10 FF | pp qq ：71 10（10x Effio 650TVL ICR DZoom） |
| 其他型号待续。。。 |  |  |  |

| 深圳市菲特数码技术有限公司 |  |  |
| --- | --- | --- |
| 地址：深圳市南山区西丽镇牛成村牛成路221号5楼 联系人： |  |  |
| 电话：0755-26414937 传真：0755-86181786 |  |  |
| 手机： QQ: Email： 网站：www.fitech.cn |  |  |